# Part 1, core experiments (design)

Design-only revision, 2026-09-01, on the PI's request: each experiment lists scene, arms, what is varied, what is measured, and code. Scene images are captures from Newton's own viewer (ViewerGL): what is shown is what the simulator draws.

Reading key:

Two contact solvers: MuJoCo (solref constraint contact) and ICF (convex contact with a literal stiffness k in N/m)

Two stepping modes each: FIXED (one chosen step size δt) and ERROR CONTROL, EC (step-doubling estimate, per-world step adapts so position error stays below a requested tolerance ε)

CENIC = ICF + error control

Smaller δt or smaller ε: more accurate, more expensive

Contact fairness: every arm, MuJoCo included, consumes the SAME Newton collision pipeline contact set; the solver is the only difference between arms

Clutter worlds: every parallel world draws its own initial arrangement (per-world seed, rejection-sampled so no bodies overlap at spawn; the same shared generator feeds the Drake harness, so CPU and GPU integrate identical world sets)

## Experiment 1, Contact stiffness realization: penetration vs step and vs tolerance

Scene (Newton viewer): the 65 g sphere at rest on the plane.

Design:

Scene: one sphere, r = 2.5 cm, density 1000 (65 g), resting on a flat floor; friction 0.5; contact margin 0; 3 s settle before readout

Contact stiffness: k = 100,000 N/m for the whole experiment (companion rows at k = 1,000 in the same CSV); ICF takes k directly; MuJoCo takes solref τ = 2.4 ms, calibrated so the converged rest depth equals mg/k at a 1 ms step (τ swept at fixed 1 ms until depth = mg/k)

Arms: ICF fixed, MuJoCo fixed, ICF EC (CENIC), MuJoCo EC

Varied, panel (a): fixed step δt in {10, 5, 2, 1, 0.5} ms; varied, panel (b): tolerance ε in {0.1 ... 0.00001} m; nothing else changes between runs

Measured: resting penetration divided by mg/k (1 = the model); a launched sphere is marked unstable

Built-in control: at δt <= 1 ms both solvers read 1.00 within 2 percent, which is the definition of same physical system (equality by convergence, no parameter translation anywhere)

Code:

step and tolerance grids, https://github.com/mardigiorgio/icra2027/blob/48604cf33b/part1/bench/benchmarks/part1_stiffness_sweep.py#L40-L41

the calibrated MuJoCo solref anchor, self-verifying at fine steps, https://github.com/mardigiorgio/icra2027/blob/48604cf33b/part1/bench/benchmarks/part1_stiffness_sweep.py#L43-L47

MuJoCo solref calibration anchors, https://github.com/mardigiorgio/icra2027/blob/48604cf33b/part1/scenes/cenic_scenes.py#L66-L69

scene build, settle loop and depth readout, https://github.com/mardigiorgio/icra2027/blob/48604cf33b/part1/bench/benchmarks/part1_stiffness_sweep.py#L50-L79

the sweep cells (arms x steps x tolerances), https://github.com/mardigiorgio/icra2027/blob/48604cf33b/part1/bench/benchmarks/part1_stiffness_sweep.py#L92-L99

solref written onto every contact geom, https://github.com/mardigiorgio/icra2027/blob/48604cf33b/part1/bench/four_arms.py#L102-L112

## Experiment 2, Work-precision on the clutter scenes

Soft clutter, world 0 (Newton viewer): t = 0 left, settled at 1.5 s right.

Hard clutter, world 0 (Newton viewer): t = 0 left, settled at 1.5 s right.

Design:

Scenes: soft clutter (20 spheres, k = 1,000 N/m) and hard clutter (10 spheres + 10 cubes, k = 100,000 N/m), dropped into a 30 cm bin; each world its own randomized non-overlapping lattice; 2 s horizon

Arms: ICF EC and MuJoCo EC (curves); ICF fixed and MuJoCo fixed at 10 ms and 1 ms (dotted reference levels)

Varied: requested tolerance ε in {0.1 ... 0.000001} m; number of parallel worlds N in {1, 1024}; N = 1 rows are 3-trial medians

Measured: wall-clock seconds per simulated second; timeout at 100 s per world-second and march-budget exhaustion are marked as crosses

Held: scene geometry, masses, friction, δt_max = 0.1 s, march budget

Code:

timeout semantics (per world-second), https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_workprecision.py#L8-L16

accuracy ladder + march budget, https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_workprecision.py#L18-L21

first attempt k_Init·δt_max, https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/four_arms.py#L47

inner tolerance rule (CENIC eq. 34), https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/four_arms.py#L48

contact budgets (>= 2x measured demand), https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/four_arms.py#L62-L64

per-world no-overlap lattice generator, https://github.com/mardigiorgio/icra2027/blob/98cb3afd46/part1/scenes/clutter_lattice.py#L38-L86

MuJoCo arms on Newton pipeline contacts, https://github.com/mardigiorgio/icra2027/blob/98cb3afd46/part1/bench/four_arms.py#L120 and #L150

## Experiment 3, Wall time vs number of parallel worlds

Scene: hard clutter, as captured under Experiment 2.

Design:

Scene: hard clutter (as in Experiment 2)

Arms: all four; fixed arms at δt = 10 ms, EC arms at ε = 0.001

Varied: number of parallel worlds N = 2^6 ... 2^13; nothing else

Measured: wall time per 100 ms control boundary, median of 3 independent trials (spread drawn as a band); per-world cost = wall / N

Held: scene, tolerance, timed window (2 s after 0.2 s warm-up)

Code:

runner (per-N subprocess), https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_scaling.py#L36

warm-up handling, https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_scaling.py#L44-L47

median statistic, https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_scaling.py#L72

defaults: 2 s timed, 0.2 s warm-up, 3 trials, https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_scaling.py#L82-L89

## Experiment 4, CENIC reference implementation (Drake, CPU) vs this work (Newton/Warp, GPU): wall time vs number of worlds

Design (replaces the actuated PD push, PI 2026-09-01):

Question: how long a batch of N worlds takes to complete the scene under error-controlled CENIC, reference CPU implementation vs the GPU implementation this paper contributes

CPU arm: Drake 1.56, integration_scheme = cenic (the reference implementation, shipped in Drake); W = min(N, 96) worker processes on the idle 128-core host, each building ONE diagram and hosting its share of worlds as ~1 MB per-world contexts, so worlds beyond the core count run SEQUENTIALLY on the cores available with no artificial per-world residency cost

Lockstep batch semantics in both stacks: every worker advances each of its worlds one 0.1 s boundary, then all workers barrier; all workers start on one shared GO after building and warming up; the parent times one makespan

GPU arm: Newton/Warp CENIC (ICF EC), batched worlds; Experiment 3 scaling data plus dedicated cells at N = 1 ... 32 so both stacks share every x point

Scene: hard clutter; both stacks import the same lattice generator, world i seeded identically, so CPU and GPU integrate identical randomized world sets

Varied: number of concurrent worlds, the SAME points for both stacks, shown 1 ... 256; both stacks also measured beyond (CPU to 2048, GPU to 8192)

Measured: plain wall-clock time for the batch to complete the 2 s scene, in seconds; no normalization; lower is better

Held in both stacks: tolerance eps = 0.001, max step 0.1 s, 0.2 s warm-up then 2 s timed, contact constants (k = 100,000 N/m, Hunt-Crossley 1.0 s/m, friction 0.5, stiction tolerance 0.0001)

Measured result (seconds to complete the scene): CPU 0.86 at 1 world, 1.6 at 64, then sequential growth past the cores: 4.6 at 256, 15.9 at 1024, 31 at 2048; GPU 1.0 at 1 world, 7.8 at 256, 64 at 2048, 245 at 8192; the curves approach each other near 8192 worlds

Reading: on this 128-core host the CPU reference is below this GPU implementation at every measured point; the GPU case rests on per-world cost at large N, co-residence with the learner (no PCIe state transfer in the training loop), and typical training hosts with far fewer cores

Code:

shared lattice import and scene constants, https://github.com/mardigiorgio/icra2027/blob/98cb3afd46/part1/bench/benchmarks/part1_cenic_cpu.py#L38-L60

Drake world build and CENIC configuration, https://github.com/mardigiorgio/icra2027/blob/98cb3afd46/part1/bench/benchmarks/part1_cenic_cpu.py#L66-L135

GO-signal barrier, lockstep boundaries, makespan timing, https://github.com/mardigiorgio/icra2027/blob/98cb3afd46/part1/bench/benchmarks/part1_cenic_cpu.py#L137-L196

GPU-side runner (Experiment 3 data), https://github.com/mardigiorgio/icra2027/blob/c0279e47d8fe/part1/bench/benchmarks/part1_scaling.py#L36